# Lipson's soft robot evolution implementation details

Hod Lipson's 2013 "Unshackling Evolution" paper provides specific implementation details for evolving soft robots using the VoxCAD platform. Based on analysis of the original paper and related technical work, here are the exact parameters and implementation specifications you requested.

## The four distinct materials used

The paper explicitly states "**four materials are used**" with these exact specifications:

1. **Green voxels**: Active muscle material that "undergo periodic volumetric actuations of 20%"

2. **Red voxels**: Active muscle material that "behave similarly to green ones, but with counter-phase actuations"

3. **Light blue voxels**: Passive soft material that "are soft and passive, having no intrinsic actuation, with their deformation caused solely by nearby voxels"

4. **Dark blue voxels**: Passive stiff material that "are also passive, but are more stiff and resistant to deformation than light blue voxels"

These four materials created a heterogeneous system with two actuated materials (green and red) and two passive materials with different stiffness properties (light blue soft, dark blue stiff).

## Precise phase relationships for actuation

The actuation used **fixed phase relationships, not random phases**:

- **Green voxels**: 0° phase (baseline actuation)

- **Red voxels**: Exactly 180° out of phase with green voxels (counter-phase)

- **Actuation amplitude**: Precisely 20% volumetric expansion/contraction for both actuated materials

- **Actuation type**: Thermal expansion-based volumetric actuation, where voxels "expand and contract at a pre-specified frequency"

The paper specifically states red voxels have "counter-phase actuations" relative to green voxels, implementing a simple two-phase system where materials oscillate in perfect opposition.

## Robot shape and size specifications

Lipson used **cubic voxel grids, not spherical robots**:

- **Primary grid size**: 10×10×10 voxels (1000 total voxels)

- **Scalable resolutions tested**: 5×5×5, 10×10×10, and 20×20×20 voxel grids

- **Voxel placement strategy**: CPPN (Compositional Pattern Producing Network) queries each voxel position using x, y, z coordinates plus distance from center (d)

- **Fill strategy**: The robots did not fill entire cubic grids - the CPPN determined which voxels were present and their material types

The genome size calculation confirms this: "a 10 × 10 × 10 ('10³') voxel soft robot with 4 possible materials would have a genome size of 10³×5 = 5000 values" (5 values per voxel: presence/absence + 4 material types).

## Simulation parameters and timing

The exact simulation specifications were:

- **Population size**: 30 individuals per generation

- **Total generations**: 1000 generations

- **Independent runs**: 35 runs per experimental treatment

- **Fitness evaluation duration**: Exactly 10 actuation cycles per robot

- **Fitness metric**: "the difference in the center of mass of the soft robot between initialization and the end of 10 actuation cycles"

- **Timestep management**: Adaptive timestep calculation based on material properties in VoxCAD

- **Recommended voxel size**: 5mm (0.005m) commonly used in VoxCAD

## Actuation mechanics implementation

The **green and red voxel system worked through volumetric thermal expansion**:

**Green voxels**:

- Expand and contract at pre-specified frequency

- 20% volumetric actuation amplitude

- 0° phase reference

**Red voxels**:

- Identical 20% volumetric actuation amplitude

- Exactly 180° out of phase with green voxels

- Counter-phase relationship creates coordinated muscle-like contractions

**Technical implementation**: The VoxCAD platform used thermal expansion coefficients to drive volumetric changes. Global temperature oscillations created sinusoidal expansion/contraction patterns, with different materials having different thermal expansion responses to achieve the phase relationships.

## VoxCAD platform specifications

The underlying simulation used these technical parameters:

- **Physics engine**: Mass-spring lattice model with Euler-Bernoulli beam connections

- **Material properties**: Young's modulus (stiffness), density, Poisson's ratio, thermal expansion coefficient, friction coefficients

- **Integration method**: Numerical integration with stability-based adaptive timestep

- **Collision handling**: Built-in self-penetration prevention

- **File format**: .vxa files (XML) containing voxel positions, materials, and parameters

## CPPN encoding architecture

The generative encoding used:

- **Inputs**: x, y, z coordinates plus distance from center (d) for each voxel

- **Outputs**: 5 total - 1 binary for voxel presence + 4 continuous for material type selection

- **Material selection**: "maximum value of the 4 remaining output nodes (each representing an individual material) specifies the type of material present"

These implementation details enabled the evolution of soft robots with complex, coordinated behaviors through the interaction of precisely-timed volumetric actuations in a heterogeneous material system. The **exact 0° and 180° phase relationship** between green and red voxels was crucial for generating effective locomotion patterns, while the **four distinct materials** provided the mechanical diversity needed for evolved morphologies to achieve different behaviors.